Nighttime Lane Detection: Enhancement, Fine-Tuning, and Comparison with Published Work

Final Report

23 September 2026

# Abstract

This report presents an independent, statistically rigorous evaluation of a lightweight, lane-aware selective image-enhancement pipeline for nighttime lane detection, tested against a CLRNet (ResNet-18) detector [1] on the CULane dataset [3]. Across three implementation iterations, enhancement applied only at test time consistently underperformed an unenhanced baseline. A subsequent audit revealed that the original 500-image night test set was 100% contained in the training set; a clean, verified held-out set of 1,000 images was built to correct this. Fine-tuning the detector on enhanced training images, evaluated with the official CULane IoU@0.5 metric and paired bootstrap significance testing, shows that the original (non-fine-tuned) checkpoint suffers a real, statistically significant F1 penalty from enhancement (p ≈ 0.002), which fine-tuning neutralizes to the point of statistical insignificance but never reverses into a proven benefit. Compared against the published literature, this project's absolute F1 numbers sit well above both CLRNet's own official Night-category result and a directly relevant real-world night baseline reported by HG-Lane [2] — a gap attributed to test-set composition differences rather than genuine outperformance. The report concludes that lightweight pixel-level enhancement, evaluated rigorously, is not a reliable substitute for the learned, distribution-aware augmentation that HG-Lane implements.

Scope note: the enhancement pipeline evaluated in this report is described purely in technical/functional terms, as an input under test, without attribution to any individual or team.

# 1. Introduction

Nighttime driving is one of the most challenging conditions for camera-based lane detection: low contrast, glare from oncoming headlights, and wet-road reflections all degrade the visual cues a detector relies on. A natural, low-cost mitigation is to enhance the image itself — brightening or contrast-correcting the frame before or during detection — rather than retraining or redesigning the detector. This report evaluates whether a lightweight, lane-aware selective enhancement pipeline actually helps a modern lane detector, CLRNet (ResNet-18) [1], on the CULane benchmark [3], with the rigor increasingly expected in this line of work: verified test-set integrity, the official evaluation protocol rather than a convenience proxy, and significance testing rather than raw point estimates.

The evaluation proceeds in the order it was carried out, mirrored in the report's structure. Section 2 describes the detector, dataset, enhancement pipeline, and fine-tuning procedure used throughout. Section 3 presents the experimental results: pipeline iteration under test-time-only enhancement, a critical data-leakage discovery and its fix, corrected fine-tuning results under both a proxy metric and the official metric with significance testing, and a qualitative case-study analysis of where the measured effect actually shows up in individual images. Section 4 discusses these findings against two directly relevant papers — CLRNet [1], the detector architecture used throughout, and HG-Lane [2], a considerably more sophisticated diffusion-based augmentation approach to the same underlying problem — and states the project's limitations. Section 5 concludes.

# 2. Method

## 2.1 Detector, Dataset, and Evaluation Protocol

All experiments use CLRNet with a ResNet-18 backbone [1], evaluated on the CULane lane-detection dataset [3]. Two evaluation metrics are used across this report: an internal proxy metric (Hungarian x-distance matching between predicted and ground-truth lane points at a 20px threshold, denoted proxy F1@20px) used during early pipeline iteration for fast feedback, and the official CULane metric (lanes rasterized to 30px-wide strips, IoU@0.5, Hungarian matching between predicted and ground-truth lanes — mirrored exactly from the CULane/CLRNet evaluation source, including its ground-truth filtering) used for all final reported results. Statistical significance of any enhanced-vs-raw difference is assessed with paired bootstrap resampling (10,000 resamples per comparison), reporting a 95% confidence interval and an empirical p-value for the difference being distinguishable from zero.

## 2.2 Enhancement Pipeline

The pipeline evaluated is a four-step, lane-aware selective enhancement approach designed to brighten only the regions of a night image likely to contain lane markings, rather than the whole frame:

- Step 1 — Rough-region pre-brighten: gamma-brighten a coarse trapezoidal region-of-interest (the area a road is likely to occupy), with feathered edges. This pass exists only to help a classical detector see lane candidates; it is not the final output.

- Step 2 — Classical lane candidate detection: Canny edge detection + HSV white/yellow color thresholding + probabilistic Hough transform on the Step-1 image, fit into left/right lane paths.

- Step 3 — Targeted re-enhancement: build a narrow (~22px) mask band around the lane paths actually detected in Step 2, and apply a stronger brighten (CLAHE + gamma) restricted to that band, on the original image.

- Step 4 — Debug visualization: overlay detected lane points on the image for manual inspection (not used downstream).

Three implementation iterations of this pipeline were developed, each addressing a failure mode found in the previous one, diagnosed by manual inspection of the Step-4 debug overlays:

- v1: full-frame pre-brighten rather than ROI-limited; debug inspection showed detected lines locking onto guardrails instead of lane paint in some frames.

- v2: pre-brighten restricted to the ROI as intended; guardrail mistracking persisted, plus a new failure mode — detected lines locking onto tree-shadow/sky-glare edges.

- v3: fixed guardrail/shadow mistracking directly — required both Canny edge AND lane color to agree (previously OR’d), tightened the ROI's upper bound, stopped extrapolating fitted lines past their actual supporting segments, and required a segment to span ≥40% of the ROI height to be trusted. Testing also surfaced a sign-flipped gamma-brighten formula (v^(1/gamma) instead of v^gamma) present since v1 — once fixed, the enhanced region's average luminance rose from ~42.7 to ~112.4, confirming the brighten was finally working as intended.

## 2.3 Fine-Tuning Procedure

In addition to applying enhancement only at test time, the CLRNet checkpoint itself was fine-tuned on enhanced night images, so the detector could adapt to the enhanced distribution directly rather than encountering it only at inference:

- Training data: night-only rows from the original training list, all run through the v3 enhancement pipeline; a new training list pointed at the enhanced images (labels unchanged, since pixel-level enhancement doesn't move lane positions).

- Run 1 (short): 3 epochs at lr 6e-5 (1/10th of the original 6e-4) — a deliberately brief, conservative setting.

- Run 2 (extended): 8 epochs at lr 1e-4 (~1/6th of the original) — more epochs and a slightly higher lr, to check whether the effect from Run 1 grew with more fine-tuning.

- Both fine-tuned checkpoints, plus the original, were evaluated on the held-out set (raw and enhanced versions) with the proxy and official metrics defined in Section 2.1, results reported in Section 3.3.

# 3. Experiments and Results

## 3.1 Pipeline Iteration: Test-Time Enhancement Only

The three pipeline iterations described in Section 2.2 were first evaluated with enhancement applied only at test time (no fine-tuning), on an original 500-image night test set, using the proxy F1@20px metric:

| Image set | zero-detect | avg drift (px) | TP | FP | FN | Precision | Recall | proxy F1@20px |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| baseline (no enhancement) | - | 11.34 | 1520 | 116 | 153 | 0.9291 | 0.9085 | 0.9187 |
| enhancement v1 (full-frame pre-brighten, gamma bug) | 24.8% (124/500) | 11.80 | 1512 | 120 | 161 | 0.9265 | 0.9038 | 0.9150 |
| enhancement v2 (ROI-limited 4-step pipeline, gamma bug) | 25.6% (128/500) | 11.60 | 1515 | 119 | 158 | 0.9272 | 0.9056 | 0.9162 |
| enhancement v3 (gamma bug + mistracking both fixed) | 27.0% (135/500) | 12.23 | 1505 | 126 | 168 | 0.9227 | 0.8996 | 0.9110 |

All three versions underperformed the unenhanced baseline. Notably, v3 — the most correct version by every checkable criterion (accurate lane-region targeting, genuinely brighter pixels) — scored worse than the buggy v1/v2 (F1 0.9110 vs 0.9150/0.9162), not better. Combined with the fact that fixing implementation bugs made results worse rather than better, this pointed to a structural explanation rather than an implementation one: at test time only, pixel-level intervention pushes the image further from the distribution CLRNet was trained on, and the more accurately that intervention is applied, the further the image drifts — independent of whether the intervention targets the “right” pixels.

Note: this 500-image test set was later found to have 100% overlap with the training set (Section 3.2). The absolute F1 values above are therefore inflated by memorization, but the relative ranking (enhanced consistently below baseline, across all three iterations) still held up once re-measured on a verified-clean set (Section 3.3).

## 3.2 Test-Set Validity: Data Leakage Discovery and Fix

Before any fine-tuning work began, the original 500-image night test set used in Section 3.1 was checked against the checkpoint's own training list by filename. All 500 images — 100% — were already present in the training set. Every F1 number reported in Section 3.1 was therefore partly measuring memorization, not generalization.

Fix: a new 1,000-image held-out set was built from the official CULane test list, filtered to night-category images, and verified to have zero filename overlap with the training list. Re-measuring the baseline on this clean set dropped proxy F1@20px from 0.9187 (old, leaked set) to 0.9004 (new, clean set) — confirming the old number was inflated by leakage. This clean held-out set (culane_night_heldout_raw / culane_night_heldout_enhanced, paired by filename) is used for every result from Section 3.3 onward.

## 3.3 Fine-Tuning Results: From Proxy Metric to the Official Metric

The internal proxy metric (Hungarian x-distance matching at a 20px threshold) initially suggested an interesting reversal: before fine-tuning, raw (0.9004) beat enhanced (0.8988); after 3 epochs of fine-tuning, enhanced (0.9026) beat raw (0.9019). But extending fine-tuning to 8 epochs flipped the proxy-metric result back the other way (raw 0.9036 vs enhanced 0.9027) — and a paired bootstrap significance test (10,000 resamples) showed that none of the three checkpoints' enhanced-vs-raw differences were statistically distinguishable from zero under this metric. The apparent “reversal” was consistent with sampling noise, not a real effect.

This motivated re-running all three checkpoints under the official CULane metric defined in Section 2.1, again with paired bootstrap significance testing:

| Checkpoint | Set (n=1000) | Precision | Recall | F1 (official IoU@0.5) |
| --- | --- | --- | --- | --- |
| Original (11.pth) | raw | 0.9357 | 0.9251 | 0.9304 |
| Original (11.pth) | enhanced | 0.9321 | 0.9214 | 0.9267 |
| Fine-tuned (3 ep, lr 6e-5) | raw | 0.9334 | 0.9234 | 0.9284 |
| Fine-tuned (3 ep, lr 6e-5) | enhanced | 0.9325 | 0.9228 | 0.9276 |
| Fine-tuned_ext (8 ep, lr 1e-4) | raw | 0.9362 | 0.9245 | 0.9303 |
| Fine-tuned_ext (8 ep, lr 1e-4) | enhanced | 0.9345 | 0.9240 | 0.9292 |

Paired bootstrap significance test (10,000 resamples per checkpoint):

| Checkpoint | Enhanced − Raw F1 | 95% CI (bootstrap) | Statistically significant? |
| --- | --- | --- | --- |
| Original (11.pth) | −0.0036 | [−0.0061, −0.0013] | Yes (p ≈ 0.002) — enhancement genuinely hurts |
| Fine-tuned (3 ep) | −0.0007 | [−0.0026, +0.0011] | No (p ≈ 0.489) |
| Fine-tuned_ext (8 ep) | −0.0011 | [−0.0033, +0.0011] | No (p ≈ 0.334) |

On the official metric, the original checkpoint shows a real, statistically significant harm from enhancement (F1 lower by 0.0036, 95% CI entirely below zero, p ≈ 0.002). After fine-tuning — at both 3 and 8 epochs — that gap shrinks and is no longer statistically significant. But at no checkpoint does enhanced significantly beat raw.

The correct conclusion is therefore: fine-tuning neutralizes the harm from enhancement (removes a real, significant deficit) — it does not reverse it into a proven benefit. An earlier reading based only on the noisier proxy metric had suggested a full reversal; that reading did not survive either the switch to the official metric or significance testing.

## 3.4 Qualitative Case Studies

To understand what kind of images drive the significant harm at the original checkpoint, every one of the 1,000 held-out images was scored with the official metric on both its raw and enhanced version, and compared.

- Only 19 of 1,000 images (1.9%) show any difference at all in matched-lane count (true positives) between raw and enhanced — 981 images (98.1%) are scored identically.

- Of those 19, 16 show enhanced losing exactly one matched lane relative to raw, and 3 show enhanced gaining one — a 16:3 asymmetry consistent with the significant net-negative effect, but built from very small, per-image differences, not dramatic failures.

- Pixel-level inspection of representative cases confirmed the predicted lane positions for raw and enhanced are nearly superimposed even where a true-positive was lost — the effect is a near-threshold IoU@0.5 miss, not a visibly wrong prediction.

Case: an image where enhancement visibly degrades a previously-perfect detection

night_4067.jpg, original checkpoint. Left (raw): all 4 lanes matched (TP=4, FP=0, FN=0). Right (enhanced): one match lost and one spurious lane appears (TP=3, FP=1, FN=1).

Case: an image that looks difficult to a human but is detected perfectly by the model

night_4000.jpg. Both raw and enhanced score a perfect match (TP=4, FP=0, FN=0), despite a visually cluttered, high-glare scene. Human-perceived difficulty and model difficulty do not always agree.

# 4. Discussion

## 4.1 Comparison with Published Work

Two papers are directly relevant to this evaluation: CLRNet [1] — the detector architecture used throughout this project — and HG-Lane [2] — a much more recent, considerably more sophisticated approach to the same underlying problem (making lane detectors robust to adverse lighting/weather, including night).

4.1.1 Versus CLRNet [1]

CLRNet's own Table 1 reports F1@50 = 75.11 for ResNet-18 on the official CULane “Night” category (the standard, full test split for that category). This project's original checkpoint scores F1 = 93.04 on raw images from its own custom held-out night subset — an 18-point gap in the same direction that needs an honest explanation, not a celebratory one.

| Source | Backbone | Test set | Night F1@50 |
| --- | --- | --- | --- |
| CLRNet [1] — published Table 1 | ResNet-18 | Official CULane “Night” category, full test split | 75.11 |
| This project — original checkpoint, raw images | ResNet-18 (same arch.) | Custom held-out subset (1,000 imgs, verified 0% train overlap) | 93.04 |

This gap is almost certainly NOT evidence that this project's checkpoint outperforms the published CLRNet result. The two numbers are measured on different test sets: CLRNet's 75.11 is the full official “Night” category split (however large and diverse that officially is), while this project's 93.04 is a custom 1,000-image subset built by filtering the official test list for night-category images and confirming zero overlap with training — which addressed leakage, but was never verified to match the composition, size, or difficulty distribution of CULane's canonical Night split. A meaningful apples-to-apples comparison would require re-running this project's checkpoint directly against CULane's official Night test list, which has not yet been done.

4.1.2 Versus HG-Lane [2]

HG-Lane takes a fundamentally different, far more resource-intensive approach: a training-free, dual-stage diffusion pipeline (Canny-ControlNet for lane-geometry-preserving structure, InstructPix2Pix-ControlNet for night/dusk appearance) that generates synthetic adverse-condition images from normal CULane images, which are then used to augment detector training. This project instead tested a lightweight, classical image-processing enhancement applied either at test time or as fine-tuning input — no generative model involved.

On their own 6-category generated benchmark (their Table 1), training CLRNet-ResNet18 with their generated augmentation raises Night F1@50 from 70.09 (small dataset) / 62.67 (dataset-size-matched, unaugmented) to 91.59 — a large, one-sided improvement, on a test set of their own generated night images:

| Training configuration | Normal | Snow | Rain | Fog | Night | Dusk | F1@50 | mF1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| w/o augmentation (≈4k imgs) | 83.01 | 46.08 | 70.34 | 58.54 | 70.09 | 79.95 | 68.74 | 42.16 |
| w/o augmentation (≈24k imgs, dataset-size-matched) | 87.28 | 51.79 | 71.48 | 58.31 | 62.67 | 75.62 | 68.89 | 45.42 |
| w/ HG-Lane-generated augmentation (≈24k imgs) (HG-Lane) | 91.64 | 84.88 | 85.30 | 85.38 | 91.59 | 91.99 | 88.49 | 63.03 |

More relevant to this project is their “Real World” evaluation (their Table 5), where CLRNet-ResNet18 is tested on real, self-collected adverse-condition photos rather than generated ones. There, a CLRNet trained only on standard CULane data scores just F1@50 = 22.47 on real night images — dramatically lower than either CLRNet's own official CULane “Night” category score (75.11) or this project's held-out-set score (93.04). Training with HG-Lane's generated augmentation raises that real-world night score to 60.02:

| Training configuration | Snow | Rain | Fog | Night | Dusk | F1@50 | mF1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Trained on CULane only (no augmentation) | 35.41 | 74.78 | 50.00 | 22.47 | 20.77 | 53.03 | 25.65 |
| Trained w/ HG-Lane augmentation | 55.65 | 77.58 | 65.57 | 60.02 | 69.31 | 68.54 | 32.75 |

This large gap between CULane's official “Night” category (75.11) and genuinely novel real-world night photos (22.47, per HG-Lane's own baseline) is an important piece of context for this project: it suggests CULane's Night category, and by extension the custom held-out subset built from it, is a comparatively favorable slice of the night-driving distribution — not a stand-in for true cross-domain generalization. It also helps explain why this project's 93.04 is plausible as a within-distribution number even though it looks high next to HG-Lane's real-world baseline of 22.47.

The two approaches are not directly comparable as “winner/loser” — they operate at different points on a cost/power spectrum. HG-Lane's diffusion-based augmentation produces large, one-sided gains but requires a full generative pipeline (pretrained diffusion model, two ControlNets, careful prompt/category engineering) applied at training-data-construction time. The enhancement pipeline evaluated in this project is orders of magnitude cheaper (classical CV, no learned generative model) but, evaluated rigorously with the official metric and significance testing, does not reliably help — it measurably hurts before fine-tuning, and only reaches parity (not benefit) after. The honest reading is that lightweight pixel-level enhancement is not a substitute for the kind of learned, distribution-aware augmentation HG-Lane implements; whether a cheaper method can close some of that gap remains open.

## 4.2 Limitations

- Proxy metrics can hide or invert small real effects. The internal proxy F1 (Hungarian x-distance) was not sensitive enough to reliably detect an effect that the official IoU@0.5 metric found statistically significant — and its apparent “reversal” finding did not survive either a metric change or a significance test. Any small-magnitude claim should be checked against the intended official metric, with resampling-based significance testing, before being reported.

- Test-set leakage is easy to introduce and easy to miss. The original 500-image night set was 100% contained in the training set; this was only caught by an explicit filename-overlap check, not by any symptom in the numbers themselves.

- A verified-clean held-out set is not automatically representative. This project's held-out set has confirmed zero training overlap, but its F1 (93.04) sits well above CLRNet's official Night-category F1 (75.11) and dramatically above HG-Lane's real-world night baseline (22.47) — a gap large enough that the held-out set's match to the official Night distribution should be checked directly, not assumed.

- Enhancement effect size is small in absolute terms throughout. Every F1 difference measured in this project, across every checkpoint, is under 0.004 — the value of the official-metric + significance-testing methodology is precisely that it can tell a real small effect from noise, which raw numbers alone cannot.

# 5. Conclusion

A lightweight, lane-aware image-enhancement pipeline was evaluated across three implementation iterations and two fine-tuning regimes on CLRNet-ResNet18 [1] for nighttime CULane [3] lane detection. Enhancement significantly hurts detection F1 at the original (non-fine-tuned) checkpoint (p ≈ 0.002); fine-tuning the detector on enhanced images neutralizes this harm to the point of statistical insignificance, but never produces a significant net benefit. Compared against the published literature, this project's absolute F1 numbers sit well above both CLRNet's own official Night-category result and HG-Lane's real-world night baseline — a gap attributed to test-set composition differences that should be verified directly, not treated as a performance claim. Compared in approach, HG-Lane's diffusion-based generative augmentation [2] achieves large, reliable gains that this project's much lighter-weight enhancement pipeline does not — a reasonable outcome given the difference in technique, and a useful benchmark for how much headroom remains if this line of work continues.

# References

[1] T. Zheng, Y. Huang, Y. Liu, W. Tang, Z. Yang, D. Cai, and X. He. CLRNet: Cross Layer Refinement Network for Lane Detection. In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), 2022.

[2] D. Zhao, Q. Chen, F. He, X. Ning, and Q. Li. HG-Lane: High-Fidelity Generation of Lane Scenes under Adverse Weather and Lighting Conditions without Re-annotation. In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), 2026.

[3] X. Pan, J. Shi, P. Luo, X. Wang, and X. Tang. Spatial As Deep: Spatial CNN for Traffic Scene Understanding. In Proceedings of the AAAI Conference on Artificial Intelligence (AAAI), 2018. (Introduces the CULane dataset.)